# Copilot User’s Guide

# Updated August 2006 for Program Version 1.2

## Introduction

Copilot is an evolving program supporting the use of an A’PEXi Power FC (PFC), which is a standalone Engine Control Unit (ECU) that can be used in place of the stock ECU in many Japanese cars.    It interfaces with the Power FC through a cable (“Datalogit”) available from FC-Datalogit, which provides protocol conversion between Power FC and PC and four 8-bit analog inputs.    Datalogit ships with a program called FC-Edit.    Copilot is not meant to replace FC-Edit, although it duplicates most features and functions.    Copilot is meant to expand the possibilities of using Datalogit with PFC, with or without other, optional, interfaces.

Besides Power FC and Datalogit, you will need a laptop, tablet PC, or car computer in your car while using the program in Online mode.    To observe Air/Fuel Ratios (AFRs) or to use the Auto Tune feature, you will need a wideband O2 sensor.    Offline mode, which can be just looking at data, editing, replay, or analysis, can be done on an ordinary PC with no special hardware or software other than Copilot.    The program was developed in Visual Basic 6 under Windows XP and you should be able to install and run it under Windows NT, 2000, or XP.    It will not run properly on earlier versions such as Windows 98.

Copilot is being developed by and for Toyota MR2 Spyder owners.    You can support this continuing effort by donating an amount you see fit. For information on how to contribute, to make suggestions for improvements, and technical support, visit www.spyderchat.com and look for the Copilot Download topic in the Performance Modifications forum.    Contributions, which are strictly optional, will help improve the program and thereby the community of MR2 Spyder and other Toyota (e.g., Celica and Matrix) owners using Power FC.

This User’s Guide is also under development.    There are no figures because the appearance of the user interface is going to change and time spent documenting the current appearance would be better spent improving the program.    Descriptions and examples are minimal at this point because of the desire to make a preliminary User’s Guide available quickly, so people can get started using the program.

WARNING:

As it states in the startup dialog, the driver should not operate or become distracted by Copilot.    The program should only be operated by the passenger (“copilot”, if you will) while the car is in motion.    You might think that it is OK for the driver to operate the program while the car is parked, but local or state law may even forbid the driver from being able to see the display.    I am not a legal expert, so I cannot give any guidance on this matter, but out of concern for the driver’s and passenger’s safety and compliance with whatever laws are in effect in the user’s state or country, the program is designed to run unattended while connected to the Power FC.    It need only be started once set up initially (one time) and will connect and start logging data automatically.    Any failure of the program to perform unattended does not change this.

## Capabilities

- Data logging

Copilot logs data to a human-readable text file in Comma Separated Value (CSV) format, which means that logs can be opened by Excel (select Files of type: Text Files (*.prn; *.txt; *.csv) in the Excel Open dialog box or double-click the .csv file) for charting/graphing.    The format differs from that used by FC-Edit for the following reasons.

- Copilot logs may contain additional data that does not apply to FC-Edit, for example accelerometer and GPS data.

- Copilot is currently unable to monitor and log all the data that FC-Edit does, some of which may not even be useful.

- A different approach was taken to the issue of sample times.    Whereas FC-Edit saves a line of text containing the elapsed time in seconds and all information it can log, Copilot is designed to allow any subset of the parameters to be logged very shortly after obtaining the sample(s).    Missing parameters are indicated by two or more commas in a row or early termination of the longest possible string (missing commas and data towards the end). This format also supports adding more parameters to be logged if and when additional sensors such as an accelerometer, or more Power FC data, are added.    This will maintain backward compatibility with the user’s old Copilot data logs.

The following parameters are (or can be) logged with the current version:

Power FC data (or derived from Power FC data) and standard sensor data

- Map Indices (the row and column in effect, which are determined my load, RPM, and Map Reference settings)

- RPM

- Speed (always logged in MPH, converted to km/h for display if Metric units is selected)

- Water Temperature (temps are always logged in deg F and converted to deg C but are displayed in both units)

- Air Temperature (same comment as above)

- Airflow (Volts)

- Injector Duty (Percent)

- Injector Pulse Width (ms)

- Inj +/- (not currently converted properly… a bug)

- Ignition Timing (deg)

- Knock

- Throttle Position (percent of max VTA V)

- Gear (0 if unknown,    clutch engaged, or shifting or 1-6)

- Number of analog wideband O2 samples (the ones read from Datalogit)

- Analog AFR (read from Datalogit)

- Number of digital wideband O2 samples (the ones read from LC-1 serial interface)

- Digital AFR

- Throttle Position Sensor (VTA Volts)

- PIM (Volts)

- O2 Sensor 1 (narrowband O2 sensor input, Volts)

- OS Sensor 2 (as above)

- Boost (PSI, converted to kg/cm^2… both displayed).    Boost is probably always going to be 0.

Optional sensor data

- Lateral G force (optional hardware: Phidgets dual axis accelerometer ($63.75)).    The current version of Copilot supports just one accelerometer and one axis is used for less-than-perfect mounting and roll compensation (meaning that your mounting can be imperfect… there is a Calibrate button).    If Copilot detects the presence of a Phidgets accelerometer, it will log the data and allow you to open the G Force floating window.    Support for a second accelerometer may be added in the future.    See The Future of Copilot section.

http://www.phidgetsusa.com/cat/viewproduct.asp?category=4000&subcategory=4100&SKU=1053

- Oil Temperature (optional hardware: oil temperature sensor).    I use this one ($58):

http://www.race-technology.com/WebPage2/Products/Sensors/OilTemperature.html

Note: Microsoft Excel, when used in some countries, expects spaces instead of commas for delimiters between fields.    This may prevent CSV files produced by Copilot from being processed correctly by Excel in those countries, unless the user converts commas to spaces.    This presents another problem, however.    Since real numbers are used in logs for some values, when Copilot is run on a computer in some countries, the real number text format may contain commas in place of decimal points.    Refer to Known Bugs in the Limitations section.

- Editing and/or display of the following data:

### Power FC data

- Platform and Version – display only

- Base Map (current, new, and default)

- Injector Correction (“Inj”) Map (current, new, and default)

- Ign Map (for 2ZZ-GE there is just one, for 1ZZ-FE there are two (Leading and Trailing)

- VVT-i Map (current and new)

- Rev Limit and Idle Settings

- "Function Select" Booleans (Boost control kit, Injector Duty and Airflow warning, Knock warning, O2 feedback control, Idle ignition control)

- O2 feedback setting (usually 1.047) -- display only

- Anything that can be logged -- display only

Other map type data (not stored in Power FC)

- Target Air/Fuel Ratio (AFR) map

- Target Degrees from Knock map

- Actual (logged) AFR map (min, max, average, and number of samples can be viewed)

- Actual (logged) Knock map (min, max, average, and number of samples can be viewed)

- Minimum Base Map

- Maximum Base Map

Display and comparison options, with copy and paste allowed between maps whenever this makes sense:

- New Base map vs Current Base map

- New Inj map vs Current Inj map

- Logged AFR map vs Target AFR map

- New Base map * New Inj map

- Current Base map * Current Inj map

- Default Base map * Default Inj map

- New Base map vs Default Base map

- Current Base map vs Default Base map

- Default Base map

- Default Inj map

- Default AFR map (just a different way of viewing the Default Inj map… Default Base map is supposedly set up to give 14.7 AFR everywhere on a stock engine and Default Inj map commands other AFRs)

- Current Ign map vs New Ign map (Leading and/or Trailing)

- Current VVT-i map vs New VVT-i map

- Auto Tuning to match Target AFRs:    Analysis of current base map, actual AFRs and other logged parameters, desired (“Target”) AFRs, and calculation of changes to the Base Map using user-defined settings.    Settings include:

- Number of samples required in each cell for it to be considered

- Number of cells with sufficient data to be changed that will cause Online Auto Tuning (advanced topic)

- What percentage of calculated minus current base map difference to use in determining new base map value (used to control how quickly AFR approaches target AFR)

- How far from target AFR is “close enough”

- Whether or not to change cells outside of the defined (minimum and maximum) range for that cell when Offline Auto Tuning.

- “Auto Tuning Required Conditions”: mostly minimum and maximum values for different parameters that must be met before an AFR sample is to be considered:

- Min and Max Water Temp

- Min and Max Air Temp

- Min and Max Airflow (percent of max possible)

- Min and Max Injector Duty (percent)

- Min and Max Injector Pulse Width Percentage of Base Map Value (very useful… prevents very lean AFRs sampled during deceleration from being considered)

- Min and Max Thottle Position (percent of max possible)

- Min and Max Throttle Accel (percent per second)

- Min and Max Gear

- Holdoff (minimum time in a cell before an AFR sample is considered)

- Auto Tuning Ignition Timing

- Per-cell timing advance until a user-defined (per cell) knock level is reached or exceeded

- Per-cell timing retard when a user-defined (per cell) knock level is reached or exceeded

- Launch control

- Automatic timing from any speed to any other speed.

- Graphing of the following parameters:

- Speed

- RPM

- RPM/Speed (useful for determining what gear is/was selected)

- RPM Increase Rate (RPM/sec)

- Airflow %

- Intake Air Temperature

- AFR

- Power FC data is read back after being written and the checksum is verified.    The program also checks for Acknowledge (ACK) or Negative Acknowledge (NAK) or timeout after write commands.    All writes to the Power FC, even in Auto Tune mode, must be initiated by the user.    Future versions may support automatically updating the Power FC after editing.

- Current and New Power FC maps (Base, Inj, Ign, VVT-i) are kept separate and can be loaded and saved separately.    They can be compared with differences displayed as difference (subtracting current from new) or percent change.    You can therefore temporarily rename your “New” files (e.g., NewBaseMap.pfc), copy someone else’s map into the folder you are working in with the appropriate “New” name, run Copilot, and compare them.    You can do copies such as copying NewBaseMap.pfc to BaseMap.pfc or the reverse when working offline.    Note that copying BaseMap.pfc to NewBaseMap.pfc will mean that the extra information at the end of the NewBaseMap.pfc file will not be there.    See next bullet.

- Extra information is kept at the end of “New” files.    This includes, for each cell, flags that indicate whether or not the cell is:

- Protected

- Analyzed

- Filled

- Smoothed (Smooth function is not implemented yet)

- Minimum Limited

- Maximum Limited

Notes relating to flags: When doing Auto Tune in multiple phases, you will use the File  New Auto Tune Folder function, which will copy all the maps in the current Auto Tune folder into the new one, so the extra information will be preserved.    This way, you will always have a running account of which cells were analyzed.    New analysis results, new cells that it has enough data for, will have their “Analyzed” flag set.    When looking at a New map, you can choose to display all cells or just the cells that fall into the same category.    You can clear certain flags individually or all flags.

- Cells may be copied and pasted.    Any rectangular area can be selected and all cells set to the same value or changed by the same value, with per-cell limit and “protection” checks.    You can copy one cell into any number of destination cells, any number of complete columns into any number of complete destination columns, any number of complete rows into any number of destination rows, or any rectangular area into the same shape area in the destination.

- Ability to protect individual cells in new maps, with protection indicated by a different color text.    The color depends on which map it is, as different maps have different color-coding schemes.    When a cell is protected, this means that the program will not write to it, be that a paste, edit, fill, or analyze operaton..    The cell will still be written to the Power FC when you do a write.      The thing to do when starting out is to make sure the new map is unprotected, copy the current map to it, then protect cells you do not want to be changed, for example the first row or two, which are mostly for starting and idling.

- All cells are color-coded.    For example, AFR data is color-coded in the same way as the AFR gauge, with numbers near stoichiometric being yellowish, lean numbers being towards red, and rich numbers being towards green.    The color is continuously variable, but there are “steps” at plus and minus 2 from stoichiometric.      Other color coding schemes are linear and either vary from red to green, red to cyan, or cyan to red.

- Cell border color varies to indicate something of importance:

- In the Injector correction map, cells with a magenta border are for closed loop (Inj value less than O2 feedback setting).

- In the Base Map, a cell which the auto tuning function would have changed if not for the minimum base value for that cell is shown with a green border.

- In the Base Map, a cell which the auto tuning function would have changed if not for the maximum base value for that cell is shown with a red border.

- Automatic loading of “Default” maps into program memory from files at startup for comparison and copy purposes.    Any Base and Inj maps can be copied to these files.

- Programmable Alerts -- visual and/or audible warning of unusual conditions (high or low temperatures, maxxed injectors, high knock, etc.).

- GPS data display,    logging, and replay (requires GPS receiver)

- Latitude and longitude (logging of position can be disabled for privacy)

- Heading

- Speed

- Altitude    (not logged)

- Time (UTC)    (not logged)

- Fix quality and Number of satellites (not logged)

- Support for Innovate Motorsports LC-1 Lambda Cable:

- Digital data acquisition using the LC-1 serial cable, providing status and preprocessed (smoothed/averaged) AFRs.

- Automatic setting of analog input conversion constants to match LC-1 analog out factory settings

- Ability to log both digital and analog AFRs and decide later which of them to display and analyze

- Display of Lambda and current multiplier programmed into the LC-1

- Calibration

- Display of LC-1 status:

- Normal

- Extremely Lean (displays %O2 instead of Lambda)

- Warming Up

- Heater Calibrating

- Free Air Calibrating

- REQUIRES CALIBRATION

- ERROR CODE (with #)

- Virtual Instruments:

- Tachometer with:

- 11,000 RPM display range

- Display of currently programmed rev limit to max display range in red

- Display of currently programmed lift on RPM to rev limit range in green (when water temperature allows lift) – only if your Power FC is the 2ZZ-GE model

- Digital as well as analog (needle) display of RPM

- Display of the following parameters as numbers and bar graphs:

- Knock

- Injector Duty %

- Airflow %

- Air Temp (appropriate units)

- Speedometer with:

- 150 MPH range (approx. 242 km/h)

- Selectable units

- Digital as well as analog (needle) display of speed with accuracy to less than 1 MPH

- Display of Throttle Position (% of max) as a number and bar graph

- Air/Fuel Ratio (AFR) gauge with:

- Digital AFR readout (range can exceed 7.35 to 22.39 when LC-1 input is selected), color-coded based on AFR (as are all AFR displays)

- Moving “illuminated” segment, like the LEDs on some AFR gauges, color-coded

- Combination Water/Oil Temperature gauge (currently just a digital readout for each) with appropriate units

- Accelerometer (floating window) with:

- Roll compensation

- Calibrate button (compensates for mounting angle, altitude, and sensor)

- Peak hold (click peak number to reset)

- Digital and bar graph display of lateral G

## Limitations

- Copilot currently supports just the 1ZZ-FE and 2ZZ-GE engines and there are no plans to support others.

- It reads and writes a subset of all the data that it possibly could using Datalogit.    In general, if there is not a need for Copilot to access the data, it won’t.    In the future, after more analysis, coding and testing is done, it will access more data to improve accuracy, utility, and convenience.

- Known Bugs:

- Sometimes the Power FC stops responding to data logging commands.    Data logging commands are read commands, which are sent periodically to read data that is used to update dynamic displays and is saved in a Copilot Data Log.    Sometimes the program is able to detect this and recover by one of several possible responses and sometimes it is unable to detect this and/or recover the ability to communicate on its own.      The easiest way to tell if Copilot is communicating with the PFC is to observe the read and write status “lights” at the very bottom right corner of the Copilot window.    If the lights go out and remain out, click the Restart Comm button.    See the WARNING in the Introduction.    This problem only happens rarely.    I have had no communications dropouts for hours at a time.

- Limited “international awareness”.    There are differences in real number formats between some countries and other international formatting issues may (probably will) cause problems.      Users in those countries may have to wait for a later version of the software unless they can work around it by changing their operating system settings to be like the United States.

## Getting Started

Whenever you start the program, either in the car or not, either physically connected to the Power FC via Datalogit or not, you will be given a choice as to how the program is to be used (modes, if you will).      Select Auto Tune or Manual Edit.    This will determine a number of things: which functions can be performed, which buttons and menu items will appear, and what folder is used for files (maps and data logs).      Check or uncheck the Work Offline box.    If you check Work Offline, you will not be able to connect to the Power FC, even if it is physically connected, but you will be able to use the auto tuning analysis function if Auto Tune is selected.    If you check the Replay box this will automatically check Work Offline and you cannot change it.    You cannot replay until you have logged data or have someone else’s logged data and other files.    But let us assume that you are starting from scratch and have to create your own files.

Before working offline for the first time, you should work online so Copilot can read all the maps.    It tries to read maps from somewhere (files and/or Power FC) every time it starts, regardless of mode.    It first tries to read all the maps in the folder you are in.    Under certain conditions (nor during Replay) it will copy the current map files to the new maps files.    This will happen if a new map file is not found or contains invalid data.    As soon as Copilot connects to the Power FC, it will read all the maps and update the current map files.    Before it can connect, the following conditions must exist.    This is the same as for FC-Edit, by the way.

Datalogit connected to Power FC and PC or laptop

Comm Port defined

Ignition On (car need not be started)

If you have not already specified the Comm port on the computer you are using (it is saved in the registry, so you have to do this for each machine you use), select Comm  Settings, Select the Comm Port, the click Apply, Save As Default, and OK.    Do not change anything else on the Comm Settings window.    In case you do and need to recover, the following settings have been used successfully:

Baud Rate: 57600

Data Bits: 8

Parity: N/A

Stop Bits: 1

Flow Control: none

DTR Enable: checked

RTS Enable: checked

Next, select Comm  Connect.      It should connect and you will see the status lights at the bottom right corner of the main window light while maps are being read.    If you get a Connect Error message box, click OK, then try a different Comm Port.    Try running FC-Edit and seeing what Comm Port it uses.

Before going much further, there is a scrolling window that can be useful for verifying events and checking for error conditions.    Select View  Message Log to bring up the window.    Use the Log menu to Clear, Save, or Print the log and the Detailed Information menu to enable/disable different types of messages.    Enabling all the types of messages will slow down execution somewhat and you should probably avoid this unless you are experiencing problems.    Error messages, for example failure to connect and communications timeouts, are displayed in bright red.    Warning messages are displayed in dark red.    If you save the log, it will be saved as a rich text file with colors preserved).    All error messages contain “Error” and all warning messages contain “Warning” to make searching for them easier.

When working online and connected, you can click the Read (whatever) Map buttons at the bottom of three tabs that display maps.    There are two read buttons on the Fuel Inj / AFR tab, one for the Base map and one for the Inj map.    If the read map buttons are not enabled, it is either because there is no connection or the program is not in the “Ready” state (see status bar at bottom of main window).    Note: You are prevented from writing maps while data acquisition is taking place.    This restriction may be lifted in the future.    Data acquisition starts when you click Start Acquisition or have enabled Auto Start Acquisition After Connect on the Setup tab.    It is a good idea to check that box after setting up the Comm Port successfully.    Please note that using a different USB cable or plugging it into a different USB port on your computer or hub may require that you change the Comm Port assignment.    Assuming you have the program connected, and all maps have been read, it is a good idea to go to the Setup tab and make sure the Auto Start Acquisition After Connect    and Auto Reconnect on Timeout or Error boxes are checked.

## Examples

To be specified later.

## The Future of Copilot

This section is dedicated to outlining future plans for Copilot.    These planned improvements may or may not be implemented. Much depends upon available time to spend on the project.    Besides correcting bugs and improving this manual, plans include (in no particular order):

- Mapping and replay of the car’s course… intended for road racing or autocross use.

- Performance analysis/measuring and display, such as graphs of torque and HP, 0-60MPH, and ¼ mile.

- Auto Tuning improvements

- Fully automatic determination of gear (it is presently reliant on initial use inputs)

- Speedometer correction (the virtual speedometer, not the real one in the instrument panel).

- Improved trip meter.

- More control over Replay, for example allowing going to a specific point in time, replaying in reverse.

- Synchronization of replayed instruments and other data with audio and/or video… intended for road racing and/or autocross.

- Support for additional analog inputs as well as digital inputs and outputs via a USB data acquisition interface (another Phidgets product).

- Improving the user interface (better controls and displays).